| 1 928 404 916 58 way Female CONN. | S.Seal 282080-1 2way Male CONN. | Description |
| --- | --- | --- |
| 211 | 1 | CAN L |
| 224 | 2 | CAN H |

| 1 928 404 916 58 way Female CONN. | S.Seal 282080-1 2way Male CONN. | Description |
| --- | --- | --- |
| 209 | 1 | CAN L |
| 222 | 2 | CAN H |
| Conn.Programmer Cable |  |  |

| 1 928 404 781 96 way Female CONN. | Mini-Fit6way Male CONN. | Description |
| --- | --- | --- |
| 143/VBAT-(Ref.B1) | 1 | GND |
| 140 | 2 | OSSD 2.2 |
| 116 | 3 | OSSD 1.2 |
| VBAT+(Ref.B1) | 4 | 24V |
| 139 | 5 | OSSD 2.1 |
| 115 | 6 | OSSD 1.1 |
| Conn.Ultrasuonic ECU |  |  |

| 1 928 404 781 96 way Female CONN. | S.Seal 282088-1 4way Male CONN. | Description |
| --- | --- | --- |
| 158 | 1 | Signal FWD |
| 142 | 2 | GND |
| 182 | 3 | Signal REV |
| 118 | 4 | GND |
| Conn.Output FWD/REV |  |  |

| 1 928 404 781 96 way Female CONN. | S.Seal 282088-1 4way Male CONN. | Description |
| --- | --- | --- |
| 141 | 1 | Signal REV |
| 117 | 2 | Signal FWD |
| 143/VBAT-(Ref.D1) | 3 | GND |
| VBAT+(Ref.D1) | 4 | 24V |
| Conn.Output FWD/REV |  |  |

| Data | Descrizione |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 10/06/2022 | Pinout lato centralina SIMAI |  |  |  |  |  |
| Diseganto/Designed by R.Mucci | Verificato da/Verified By Approvato da/Approved by N/D N/D |  |  | N°Foglo/N°Sheet 1/4 |  |  |
|  | Note/Notes Schema elettrico centralina HTTC 540 con sensore ultrasuoni e radiocomando |  |  |  | Rev/Rev 0 | Codice/PartNumber Da Richiedere |

| Data | Descrizione |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Diseganto/Designed by | Verificato da/Verified By Approvato da/Approved by |  |  | N°Foglo/N°Sheet |  |  |
|  | Note/Notes |  |  |  | Rev/Rev | Codice/PartNumber |